Currently making a plan to work on LCD driver

https://leeselectronic.com/en/product/5576-lcd-display-4x16-char-tc1604a-01.html

Data from datasheet

https://dosya.motorobit.com/pdf/TC1604a.PDF

Pin setup

VCC,V0,VSS  - controls brightness, can keep at 3.3V can dim by attaching potent or resistor

DB0-DB7: – 8 GPIO’s needed

RS – 1: Data register for read and write, 0: Instruction register (for write)

R/W - 1: (Read mode, 0 Write mode)

### Initialization sequence

Power on

If VDD>4.5 wait 15ms

Send for 8-bit length interface

Wait 100us and send again

AFTERWARDS

Display Off - 0 0 0 0 0 1 0 0

Display Clear: 0 0 0 0 0 0 0 1

Entry Mode Set: 0 0 0 0 0 0 1 I/D S

I/D = 1 → Cursor moves right after each character.

I/D = 0 → Cursor moves left after each character.

S = 1 → Display shifts instead of the cursor.

S = 0 → Display does not shift.

### Sending characters:

It should automatically shift DRAM data

# DRAM DATA MEMORY address

# CHARACTER GENERATION

EX) LLLLHLH -->0000 0101 --> corresponds to the letter P

Creating temporary module.

Timing calculations

MINIMUM TIMING is 100us,  f=10000Hz

5MHzdivider=100000

divider=500

I will choose 29=512 , which gives 9765.625Hz or 1.024*10-4 = 102.4us

Power on wait time atleast 15ms.

0.015s=N*102.4*10-6s

N=146.48=150.

Initialization sequence

Pinouts:

## E pin –  assumption: All commands are registered through e trigger. So we can actually put that as the clock.

## R/W pin –

Read mode(lcd -> MCU - 1)

write mode (mcu -> LCD - 0) – don’t we always keep this on?

## RS pin:( selects registers)

1 – data register <- I believe we are doing this

0 - instruction register or busy-flag address counter

Keep both these registers low, except for the following commands.

Afterwards we are going to be writing to MCU R/W -> 0

RS-PIN UNKNOWN what is data-register, vs instruction register

Instruction register

Data register is also known as RAM. Hard to know, I can try both methods.

But for now RS-> 1 for sending data.

Figure 1 Location of writing characters